(Translation)

NOTIFICATION OF REASON(S) FOR REFUSAL

| Applicant(s) : | DYSON TECHNOLOGY LIMITED |
| --- | --- |
| Attorney(s) : | Yoon & Yang (IP) LLC |
| Application No. : | 10-2023-7038451 |
| Title of the invention : | A HEATER |

- With reference to the above identified application, you are hereby notified of the ground(s) for rejection, as mentioned below according to Art. 63 of the Patent Law. The applicant may file a written argument and/or an amendment by the designated date.

- Please be informed that any request to extend the deadline to respond to an Office Action (O/A) is now limited up to four (4) months beyond the original deadline upon pre-request and payment. If necessary, the extension can be pre-requested by one or more months up to a maximum of four months. Additional term extensions exceeding four (4) months owing to unavoidable circumstances (refer to guidelines below) are obtainable, however, only by submitting a written statement explaining the necessity for the additional term extension.

Result(s) of Examination

- Examined Claims: 1 to 21

| No. | Rejections | Basis for Rejection |
| --- | --- | --- |
| I | Claim 7 | Article 42(4)(ii) of the Patent Act [Lack of Clarity in the Claims]* |
| II | Claims 1 to 14 and 16 to 21 | Article 29(2) of the Patent Act [Lack of Inventive Step]* |

- Rejected Items and Applicable law

- Patentable Claims: Claim 15

※ The patentability of claims is determined based on the examination at the time the outstanding Office Action is issued and may be subject to change at a later time.  All grounds for rejection must be overcome in order to receive a patent for the invention claimed in the subject application.

GROUND I

Claim 7 of the subject application is unclear for the reason set forth in the “Remarks” section below.  Accordingly, the claim cannot be allowed pursuant to Article 42(4)(ii) of the Patent Act.

REMARKS

Claim 7 of the subject application includes the expression “in particular” which makes the invention described in the claim unclear.  Therefore, the invention in Claim 7 fails to comply with the clarity and conciseness requirements for the patent claims.

GROUND II

The inventions defined in Claims 1 to 14 and 16 to 21 of the subject application could have been easily conceived by one of ordinary skill in the art prior to the filing of the subject application for the reasons set forth in the “Remarks” section below.  Accordingly, the claimed inventions cannot be patented pursuant to Article 29(2) of the Patent Act.

REMARKS

D1:	US 2015/0139681 A1  (May 21, 2015)

D2:	KR 10-2019-0141227 A  (December 23, 2019)

D3: 	KR 10-2011-0042746 A  (April 27, 2011)

2-1. Claim 1 (Independent Claim)

A. Element Comparison Table

D1 is an invention related to a heater and heater connector, which is highly technically relevant to the present application, and the elements of Claim 1 of the present application correspond to the elements of D1 as follows:

[Elements Comparison Table]

| Main Elements | Claim 1 | D1 | Remarks |
| --- | --- | --- | --- |
| Element 1 | A heater coupon comprising a ceramic block having a first surface and a second surface, at least one heater trace formed within the ceramic block and a connector for electrically connecting the at least one heater trace to an external wire | A heater (5) comprising a dielectric substrate (9) made of ceramic material having first and second surfaces, a heating element (8) formed on the upper part of the dielectric substrate (9), and a connector (13) for electrically connecting the heating element (8) to an external wire (paragraphs [0048 to 0049], [Figs. 2, 3, and 4]) | Partially different |
| Element 2 | Wherein the connector comprises a first connecting arm for connecting with respect to the first surface of the ceramic block and a second connecting arm for connecting with respect to the second surface of the ceramic block. | The connector (13) includes an upper and lower housing (15) that surrounds the center recess of the connector (13), which includes spring contacts (16f, 16r), in order to connect with electrodes (10 and 10f, 10r) of the first and second surfaces of the dielectric substrate (9) (paragraphs [0055 to 0058], [Figs. 4, 5, and 6]) | Identical |

B. Review of the Differences

As shown in the table above, Claim 1 is characterized by the heater trace formed within the ceramic block, which is different from the features of D1 which discloses that the heating element (8) is formed on the upper part of the ceramic dielectric substrate (9).

However, D2, which is an invention related to an electrical apparatus comprising a ceramic heater element (paragraph [0014]), discloses that an electrical resistance track, which is a ceramic heater element, is disposed between layers of ceramic material.  Therefore, the above difference could be easily derived by a person skilled in the art.

Meanwhile, D1 and D2 are both inventions related to ceramic heaters and thus pertain to a common technical field, so there is no particular difficulty in combining D1 and D2.

Therefore, Claim 1 could be easily derived by a person skilled in the art from a combination of D1 and D2.

2-2. Claims 2 to 4, 13, 14, and 21

The technical feature of Claim 2 could be easily derived by a person skilled in the art from the cited references as D1 (paragraph [0054], [Fig. 4]) discloses that the connector (13) is connected to the longitudinal end of the heater (5).

The technical feature of Claim 3 could be easily derived by a person skilled in the art from the cited references as D2 (paragraph [0014]) discloses that an electrical resistance track, which is a ceramic heater element, is disposed between layers of ceramic material.

The technical feature of Claim 4 could be easily derived by a person skilled in the art from D2 (paragraph [0040]), which discloses that the ceramic heater element is a multilayer ceramic substrate, and this substrate includes a resistance track that is printed on the inner layer while the substrate is in green state.

The technical feature of Claim 13 could be easily derived by a person skilled in the art from D1 ([Figs. 2, 3, and 6]), which discloses that “among the connector electrodes (10f, 10r), two electrodes (10) at the other longitudinal end of the heater support member (6) are connected to two separate connectors (13) so that from the perspective of electrical circuitry the heating unit (2) of the present embodiment is configured to allow the two heating elements (8) to be independently controlled using three connectors.”

The technical feature of Claim 14 is not disclosed in D1 or D2, but a solder cup is a cup-shaped contact or terminal designed for reliable soldering of the end of a wire, which is a structure commonly used in wire end connectors or terminals.  Accordingly, the feature of Claim 14 could be easily conceived and derived by a person skilled in the art in the technical field of the application.

The technical feature of Claim 21 is not disclosed in D1 or D2, but a strain relief is a component used to prevent the soldering portion of the wire from easily breaking as the wire moves in the cases where the wire is directly soldered to the terminal, which is also a commonly used structure.  Accordingly, the feature of Claim 21 could be easily conceived and derived by a person skilled in the art in the technical field of the application.

Therefore, Claims 2 to 4, 13, 14, and 21 can be easily derived by a person skilled in the art from a combination of D1 and D2.

2-3. Claims 5 to 12 and 16 to 20

Regarding the technical feature of Claim 5, D1 (paragraph [0052], [Figs. 2 and 3]) discloses that the heater support member (6) has recesses to accommodate the spring contacts (16f, 16r) arranged inside the upper and lower housing (15).  Meanwhile, Claim 5 differs from D1 in that the recesses are located in the ceramic block, whereas in D1 they are located in the heater support member.  However, when considering the multilayer ceramic of D2, it is obvious that a recess is to be formed to expose the resistance track printed on the inner layer of the multilayer ceramic.  Furthermore, D3 (paragraphs [0025, 0030, and 0035], [Figs. 4 and 5]), which is an invention in the same technical field (a ceramic heater), discloses providing first and second inlet holes (111, 121) in the second and third sheets (120, 130) made of ceramic.  Therefore, the above difference could be easily derived by a person skilled in the art based on the disclosure of the cited references.

The technical features of Claims 6, 7, 9, 11, and 12 could be easily derived by a person skilled in the art from D1 ([Figs. 2 and 3]), which discloses that the heater support member (6) is provided with a recess having a certain depth (corresponding to the first and second inlet holes (111, 121) of D3) and that the recess exposes the connector electrodes (10, 10f, 10r) which extend from and are electrically connected to the heating elements (8) formed on the first and second surfaces of the dielectric substrate (relevant to Claims 6, 9, and 11 of the present application), wherein the connector electrodes (10, 10f, 10r) located in the recess are coupled to (exposed to) the connector (13) (relevant to Claims 7 and 12 of the present application).

The technical features of Claims 8 and 10 could be easily derived by a person skilled in the art from D1 ([Figs. 2, 3, and 6]), which discloses that the recess of the heater support member (6) has a certain depth and that the recess has a wall and a tab, wherein the wall defines the certain depth and the tab provides a surface that can be connected to the spring contacts (16f, 16r) inside the housing (15).

The technical feature of Claim 16 could be easily derived by a person skilled in the art from D3 (paragraph [0025], [Figs. 4 and 5]), which discloses a ceramic heater with a built-in temperature sensor.

Regarding the technical feature of Claim 17, D3 (paragraphs [0025 to 0027], [Figs. 4 and 5]) discloses that the ceramic heater with a built-in temperature sensor forms a temperature sensor pattern (160) on an alumina (ceramic) sheet (110) and has a temperature sensor connection electrode (161) placed on one side.  Therefore, the configuration of Claim 17 can be achieved by connecting the ceramic heater with a built-in temperature sensor disclosed in D3 to an external wire through the connector of D1 (paragraphs [0048 to 0049], [Figs. 2, 3, and 4]).

The technical feature of Claim 18 can be achieved by connecting the temperature sensor of D3 (paragraphs [0025 to 0027], [Figs. 4 and 5]) to the connector of D1 (paragraphs [0055 to 0058], [Figs. 4, 5, and 6]) that has a center recess.

The technical feature of Claim 19 could be easily derived by a person skilled in the art from D3 (paragraphs [0025, 0030, and 0035], [Figs. 4 and 5]), which discloses providing first and second inlet holes (111, 121) in the second and third sheets (120, 130) made of ceramic.

The technical feature of Claim 20 could be easily derived by a person skilled in the art from D1 ([Figs. 2, 3, and 6]), which discloses that the recess of the heater support member (6) has a certain depth and that the recess has a wall and a tab, wherein the wall defines the certain depth and the tab provides a surface that can be connected to the spring contacts (16f, 16r) inside the housing (15).

Therefore, Claims 5 to 12 and 16 to 20 could be easily derived by a person skilled in the art from a combination of D1, D2, and D3.

[Annexed Document]

Attachment 1	 US 2015/0139681 A1  (May 21, 2015)

Attachment 2	 KR 10-2019-0141227 A  (December 23, 2019)

Attachment 3	 KR 10-2011-0042746 A  (April 27, 2011)

# Patent Examiners: J. H. Woo and W. L. Lee

Display Device Examination Division

Semiconductor Examination Bureau

Korean Intellectual Property Office

------------------------------------------------------------------------------------------------------------------------------

[Guidelines]

A.   Additional Term Extension

When an applicant submits a written statement explaining the necessity for an additional term extension exceeding the allowed term (i.e., a four-month extension granted once), the Examiner will review the statement and determine whether the additional term extension is granted within two weeks from receipt of the statement.

[Allowable circumstances of the statement]

1.   An applicant appoints an attorney(s) for the first time, or dismisses or changes all previously appointed attorneys within one month prior to the expiry of the full extended term (i.e., six months after the O/A was issued).

2.   A petition to change an applicant is submitted within one month prior to the expiry of the full extended term (i.e., six months after the O/A was issued).

3.   An amendment is filed to reflect an examination result, which was issued by foreign patent office within two months prior to the expiry of the full extended term (i.e., six months after the O/A was issued). In this case, upon submitting the statement, copies of the Notification of Examination Result as well as the claims based on the notification should also be submitted.

4.   The delivery of an O/A is delayed one or more months.

5.   A parent or divisional application is pending in a trial or litigation.

6.   An applicant requires more time to perform experiments or verify measurements.

7.   Specific circumstances arise, which are not attributable to the applicant.

However, if a third party requests an examination for the application, it shall not be allowed if at least one of the circumstance Nos. 1 to 5 above is met.

B.   Prior Review Service over Draft Amendment

An applicant can request a prior review for a draft amendment no later than one month prior to the due date for filing the amendment in response to an Office Action, and through an interview with the Examiner, an applicant may discuss how to amend the application to enhance the chances of gaining its allowance.

※   The draft amendment should be submitted as a formal form of amendment or argument prior to a request for a prior review of the draft amendment.